Arduino编程及应用开发·课程作业#3

| 姓名： | 林新杰 | 学号： | 202536210121 | 班级： | 光电251 | 日期： | 2026.9.7 |
| --- | --- | --- | --- | --- | --- | --- | --- |

【要求】

用1个数码管和2个按键，设计一个按键控制的计数器，计数范围0~9，用按键A控制每次加1，用按键B控制每次减1，用1位数码管显示结果。注意按键去抖动。（成绩分：4分）

技能点：数字量输入、输出，按键去抖，数码管显示，函数，编程复杂度

【作业结果】

- 电路原理图(可用Fritzing、lcEDA绘制)

图1 电路原理图

2、Arduino源程序(注意编程规范，包括缩进、注释、命名、分段等)

- #include <Arduino.h>

- int num = 0;

- void setup() {

- pinMode(2, OUTPUT);

- pinMode(3, OUTPUT);

- pinMode(4, OUTPUT);

- pinMode(5, OUTPUT);

- pinMode(6, OUTPUT);

- pinMode(7, OUTPUT);

- pinMode(8, OUTPUT);

- pinMode(9, INPUT_PULLUP);    // 加1

- pinMode(10, INPUT_PULLUP);   // 减1

- }

- void loop() {

- // 加1

- if (digitalRead(9) == LOW) {

- delay(20);

- if (digitalRead(9) == LOW) {

- num++;

- if (num > 9) {

- num = 0;

- }

- while (digitalRead(9) == LOW) {

- }

- }

- }

- // 减1

- if (digitalRead(10) == LOW) {

- delay(20);

- if (digitalRead(10) == LOW) {

- num--;

- if (num < 0) {

- num = 9;

- }

- while (digitalRead(10) == LOW) {

- }

- }

- }

- // 初始化

- digitalWrite(2, LOW);

- digitalWrite(3, LOW);

- digitalWrite(4, LOW);

- digitalWrite(5, LOW);

- digitalWrite(6, LOW);

- digitalWrite(7, LOW);

- digitalWrite(8, LOW);

- // 显示0

- if (num == 0) {

- digitalWrite(3, HIGH);

- digitalWrite(4, HIGH);

- digitalWrite(5, HIGH);

- digitalWrite(6, HIGH);

- digitalWrite(7, HIGH);

- digitalWrite(8, HIGH);

- }

- // 显示1

- if (num == 1) {

- digitalWrite(5, HIGH);

- digitalWrite(8, HIGH);

- }

- // 显示2

- if (num == 2) {

- digitalWrite(2, HIGH);

- digitalWrite(4, HIGH);

- digitalWrite(5, HIGH);

- digitalWrite(6, HIGH);

- digitalWrite(7, HIGH);

- }

- // 显示3

- if (num == 3) {

- digitalWrite(2, HIGH);

- digitalWrite(4, HIGH);

- digitalWrite(5, HIGH);

- digitalWrite(7, HIGH);

- digitalWrite(8, HIGH);

- }

- // 显示4

- if (num == 4) {

- digitalWrite(2, HIGH);

- digitalWrite(3, HIGH);

- digitalWrite(5, HIGH);

- digitalWrite(8, HIGH);

- }

- // 显示5

- if (num == 5) {

- digitalWrite(2, HIGH);

- digitalWrite(3, HIGH);

- digitalWrite(4, HIGH);

- digitalWrite(7, HIGH);

- digitalWrite(8, HIGH);

- }

- // 显示6

- if (num == 6) {

- digitalWrite(2, HIGH);

- digitalWrite(3, HIGH);

- digitalWrite(4, HIGH);

- digitalWrite(6, HIGH);

- digitalWrite(7, HIGH);

- digitalWrite(8, HIGH);

- }

- // 显示7

- if (num == 7) {

- digitalWrite(4, HIGH);

- digitalWrite(5, HIGH);

- digitalWrite(8, HIGH);

- }

- // 显示8

- if (num == 8) {

- digitalWrite(2, HIGH);

- digitalWrite(3, HIGH);

- digitalWrite(4, HIGH);

- digitalWrite(5, HIGH);

- digitalWrite(6, HIGH);

- digitalWrite(7, HIGH);

- digitalWrite(8, HIGH);

- }

- // 显示9

- if (num == 9) {

- digitalWrite(2, HIGH);

- digitalWrite(3, HIGH);

- digitalWrite(4, HIGH);

- digitalWrite(5, HIGH);

- digitalWrite(7, HIGH);

- digitalWrite(8, HIGH);

- }

- }

- 3、实物连线图、运行效果实物照片（2~4张）

图2 实物连线图

- 调试体会

调试过程中主要遇到了数码管引脚对应关系不一致的问题，导致显示的数字不正确。通过逐段点亮数码管，重新确认了各引脚对应的 a、b、c、d、e、f、g 段，并修改程序中的引脚控制关系。

按键部分采用延时的方法进行消抖，避免一次按键产生多次计数。